| Agreed <Signature and seal> Saltanat Tompieva Aviation Administration of Kazakhstan JSC 16.02.2023 | Approved <Signature and seal> Alp Er Tunga Ersoy Almaty International Airport JSC _____ _____________ 2023 |
| --- | --- |

Technical Specification 
to the Bid Documentation for the selection of a ground handling service provider at the airport of Almaty

General Information

This technical specification was developed in accordance with Article 65 of the Law of the Republic of Kazakhstan dated 15 July 2010 “On the Use of Airspace and Aviation Activities”, by Order No. 750 of the Minister for Investment and Development of the Republic of Kazakhstan dated 02 October 2029, and defines the requirements to a list of goods, works, services for airfield and ground handling that form a part of the airport activities (hereinafter the “Services”).

List of ground handling services transferred to the competitive environment:

| Ser. No. | Section name | Name of service |
| --- | --- | --- |
| 1 | Ground administration and control | - Informing interested organizations about the movement of the carrier's aircraft; - Control over the services provided to the carrier by third parties; - Prompt transfer of data, information, requirements of the carrier to third parties; - Services of the carrier’s representative. |
| 2 | Passenger service | - Passenger service - Passenger pick-up/drop-off by means of mobile stairways |
| 3 | Baggage handling | - Baggage handling - Baggage transportation |
| 4 | Apron service | - Aircraft rendezvous/delivery; - Provision of special vehicles and equipment; - Transporting and loading/unloading baggage, cargo/mail. Loading and unloading of cargo (mail); - Aircraft towing; - Aircraft anti-icing treatment. |
| 5 | Aircraft maintenance | - Internal cleaning and disposal of aircraft debris; - Maintenance of aircraft bathrooms (toilets); - Aircraft external washing |
| 7 | In-flight catering services | - Provision of aircraft passengers and crew with in-flight meals - Sanitization of removable equipment and onboard utensils |

A potential ground handling service provider shall have a sufficient number of qualified personnel, specialized equipment and ground handling facilities for the uninterrupted handling of aircrafts operating flights to the Almaty airport, subject to the types thereof and the stated scope of services.

The Service Provider shall, at its own expense, obtain an admission to the airport airside and access to aircrafts (special passes), with the passage of all procedures for confirming the identity of personnel, through authorized special bodies, and shall also train, at its own expense, its employees, including aviation security and safety management system trainings for the operating staff. Furthermore, the Service Provider shall, at its own expense and using motor vehicles, train its personnel and obtain a permission from the Almaty airport for the right to drive on the airport apron.

The Service Provider shall conduct an independent quality control of the services provided and, at least once a quarter or upon request of the Airport, submit reports on the flight punctuality and the time of baggage first/last delivery to quaIity.ala@tav.aero.

The Service Provider shall enter into an agreement with the Airport for the provision of ground handling services and the use of the centralized infrastructure.

Types of aircraft flying to the Almaty airport:

| Airbus | A220 (-100/-300), A300 (-600R), A310, A318, A319, A321, A330 (-200), A330 (-300), A340 (-200/-300), A340 (-500/- 600), A350 (-900) |
| --- | --- |
| Antonov | A N-12, AN-124, A N-70, AN-74, AN-224 |
| Bae Systems | ATP, Jetstream 31, Jetstream 41, 146, AVRO RJ 70/85/100 |
| Beech | 1900 D, Beechjet 400 A. King Air 350, King Air B200, King Air C90B, C90SE |
| Boeing | 707, 717-200, 727, 737 (-200). 737 (-300/-400/-500), 737 (-600/-700/-800/-900), 37-100/-200/-300, 747-400, 747-800, 757-200, 767 (-200/-300/-400), 777 (200/-300), 777 -2LR/-3ER), 777 (-8/-9), 787-8, C17A Globemaster III |
| Boyin/MD | DC-10 MD-2l, MD80, MD90 |
| bombardier | 130-100 Continental, Global Express, Canadair CL600 Challenger, СL 100/200, CRJ-700, DHC-8 DASH 8 Q100/200, DHC-8 DASH 8 Q400, Learjet 31A, Learjet 45, Learjet 60 |
| Cessna | 525 Citation SJ1, 525 Citation СЈ2, 550 Citation Bravo, 560 Encore, 560 Excel, 680 Citation Sovereign, 750 Citation X |
| Dassault | Falcon 2000, Falcon 50 EX, Falcon 900B/C and 900 EX |
| EADS | ATR-42, ATR-72 |
| Embraer | ERJ 190/195, E RJ-145, ERJ-170/175 |
| Fairchild | Dornier 328 JET, Dornier 728 JET |
| Fokker | 100, 27, 50, 70, F28 Fellowship |
| Gulfstream Aerospace | IV.SP, lV-MPA and IV-B |
| Hawker | 800 XP, Horizon |
| Ilyushin | Il -62, Il -76, Il -86, Il -96 (-300), Il-96M, Il-114 |
| Let | L410, L610G |
| Lockheed | Galax C-5, Hercules C-130J |
| Mitsubishi | MU-2Ј |
| Raytheon | Premier 1 |
| Saab | 2000, 340B |
| Shorts | 330, 360 |
| Sino | Swearinger SJ30-2 |
| Tupolev | TU-334/336/354, TU-134, TU-154M, TU-204 |
| XAC | MA-60 |
| Yakolev | YAK-40, YAK-42D |

General Requirements

The Service Provider shall, pursuant to Article 64 of the Law of the Republic of Kazakhstan dated l5 July 2010 “On the use of the Airspace of the Republic of Kazakhstan and Aviation Activities”, comply with the standards established by the airport operator in accordance with the laws of the Republic of Kazakhstan on the use of the airspace of the Republic of Kazakhstan and aviation activities.

The Service Provider shall, in the course of provision of the Services, comply with the quality standards set out in the airport user manual and the international standards, as well as internal documents of the Customer, and shall also ensure the compliance with the requirements of the Sanitary Rules “Sanitary and Epidemiological Requirements to Vehicles for Passenger and Cargo Carriages”, approved by Order No. KR DSM-5 of the Minister of Health of the Republic of Kazakhstan dated 2 January 2021.

Depending on the type of the stated services, the Service Provider shall be guided by the following documents in its operating activities:

| Law of the Republic of Kazakhstan dated 15 July 2010 | On the Use of the Airspace of the Republic of Kazakhstan and Aviation Activities |  |
| --- | --- | --- |
| Law No. 567 of the Republic of Kazakhstan dated 05.07.2004 | On Communications |  |
| Law No. 94-V of the Republic of Kazakhstan dated 21.05.2013 | On Personal Data and Its Protection |
| Order No. 750 of the Minister for Investments and Development of the Republic of Kazakhstan dated 02.10.2019 | On Approval of the Rules for Ground Operations at Airports |
| Order No. 189 of the Minister for Investments and Development of the Republic of Kazakhstan dated 24.02.2015 | On Approval of the Rules for the Organizing the Passenger Service at Airports of the Republic of Kazakhstan |
| Order No. 327 of the acting Minister for Investment and Development of the Republic of Kazakhstan dated 26 March 2015 | On Approval of the Rules for Organizing Special Transport Operations at Airports of the Republic of Kazakhstan |
| Order No. 540 of the Minister for Investments and Development of the Republic of Kazakhstan dated 30 April 2015 | On Approval of the Rules for the Carriage of Passengers, Baggage and Cargo By Air Transport |
| Order No. 189 of the acting Minister for Investment and Development of the Republic of Kazakhstan dated 24 February 2015 | On Approval of the Rules for the organization of passenger service at the airports of the Republic of Kazakhstan |
| Order No. 159 of the Minister for Investment and Development of the Republic of Kazakhstan dated 24 February 2015 | On Approval of the Rules for the Professional Training of Aviation Personnel |
| Order No. 764of the Minister of Transport and Communications of the Republic of Kazakhstan dated 28 September 2013 | On Approval of Model Programs for the Professional Training of the Flight Safety Aviation Personnel |
| Order No. KR DSM-5 of the Minister of Health of the Republic of Kazakhstan dated 11 January 2021 | On Approval of the Sanitary Rules “Sanitary and Epidemiological Requirements to Vehicles for the Transportation of Passengers and Goods” |
| IATA Airport Handling Manual (AHM), Last Edition | Manual of the International Air Transport Association (IATA) |
| IATA DGR, Last Edition | Regulation of the IATA |
| IATA Ground Operation Manual (IGOM) | Manual of the IATA |
| IATA Resolution 753 | Resolution of the IATA |
| lATA Temperature Control Regulation, Last Edition | Regulation of the IATA |
| lATA Load Device Regulation, Last Edition | Resolution of the IATA |
| lATA Perishable Cargo Regulation, Last Edition | Resolution of the IATA |
| IATA Live Animal Regulation, Last Edition | Resolution of the IATA |
| IATA Cargo Service Conference Resolution Manual, Last Edition | Manual of the IATA |
| IATA Cargo Handling Manual, Last Edition | Manual of the IATA |
| IATA Standard Ground Handling Agreement | Agreement of the IATA |
| SAE AS6285, Last Edition | Standard of the Society of Automotive Engineers (SAE) International |
| SAE AS6286, Last Edition | Standard of the SAE |
| SAE ARP6257, Last Edition | Lnternational Standard of the SAE |
| SAE AS6332 | Standard of the SAE |
| ISO 20471:2013 | High visibility clothing. Test methods and requirements |
| ISO 9001:2018 |  |
| Airport User Guide |  |

The Service Provider shall maintain the valid civil liability insurance under a contract for the provision of ground handling services and the use of the centralized infrastructure for an amount of one million five hundred thousand US dollars (KZT 1,500,000).

The potential provider shall have an instruction that regulates the sequential procedures and interaction with the Department of Aviation Safety of the airport operator and the potential provider.

The Service Provider shall:

- comply with the labor protection, fire safety and industrial safety requirements;

- present an ISO 14001 environmental management certificate;

- have in place a functioning flight safety management system;

- have the developed Ground Operations Manual describing the ground handling procedures for the specified types of aircrafts that meets the requirements listed in paragraph 9 of this technical specification;

- the approved emergency plan and contingency response plan.

Resources

The provider shall that its ground operations meet the requirements of the airline, the lATA Standard Ground Handling Agreement (SGHA), the Service Level Agreement (SLA) and the airport operator standards.

The potential provider shall secure a number of qualified specialists, specialized ground handling machinery and equipment for each stated service based on the type, scope of services and compliance with the established requirements, and also provide for an additional reserve of 5% of the stated resources to ensure the business continuity;

The following minimum flight connection time shall be ensured at the airport, except for the of flights that require additional aviation security control:

| Internal – Internal 60 minutes | Domestic - International // International - Domestic 75 minutes | International - International 75 minutes |
| --- | --- | --- |

- Minimum ground time

| Aircraft type: | MGT* |
| --- | --- |
| F50 and similar | 45min |
| A 319 A320 B737 | 60 min |
| A321 B757 | 70 min |
| A330 A340 A350 | 90 min |
| B747 B777 | 110 min |
| Freighters A330 B777 | 120 min |
| Freighters B747 | 150 min |
| An 124 | 120 min |
| Preighters A330 B767 B777 | 240 min |

MGT shall be calculated for the turnover of an aircraft with full load at the entrance and exit.

* Applicable only for arrivals on time +/- l5 minutes

Requirements to the organization and management

The service provider shall have a management system that ensures that the top management of the enterprise has direct responsibility for ensuring flight safety, aviation security and the quality of the services provided, as well as its distribution and coordination throughout the organization, the availability of sufficient production and human resources to provide ground handling .

The policy of the organization shall be incorporated in the organizational documents and implemented through the prescribed procedures, technological documents to be available to the staff at workplace.

The potential provider shall have its planned and/or approved organizational structure and staffing table.

The potential provider shall have in place the flight safety management system (FSMS) and the safety management system manual (FSMM) incorporating the flight safety training system for high- and mid-level executives and the operational personnel.

The potential provider shall have an HR policy incorporating a plan for training, advanced training and admission to work of ground handling personnel in accordance with the Rules for the Professional Training of Aviation Personnel (Order No. 159 of the Minister for Investment and Development of the Republic of Kazakhstan dated 24.02.2015) ( hereinafter the “Rules”) and the standard professional training programs for the aviation personnel involved in flight safety (Order No. 764 of the Minister of Transport and Communications of the Republic of Kazakhstan dated 28.09.2013) (hereinafter the “Standard Programs”), and the requirements IATA AHM 1110.

The potential provider’s managers and personnel involved in ground operations shall be familiar with the manuals. instructions and technological documents governing the operations.

Requirements to the ground operations organization’s personnel and training thereof

The potential provider shall be staffed with administrative, managerial and operational (ground) employees required for the implementation of its activities in a number sufficient to perform the planned scope of work.

The potential provider shall have the qualified management personnel responsible for the state and functioning of the flight safety and aviation security systems.

The ground handling employees shall have certificates confirming the completion of training, as per the requirements of the Model Programs and IATA AHM 1110, issued by a training center certified by the authorized organization or a civil aviation organization, subject to the requirements of the Rules for the Professional Training of Aviation Personnel (Order No. 159 of the Minister for Investment and Development of the Republic of Kazakhstan dated 24.02.20l5).

At the time of submitting an application, the potential provider shall have at least 5 specialists with relevant certificates on its staff.

In addition to its existing staff, the potential provider may conduct recruitment and training for up to two months from the date of execution of a contract.

The potential provider's organization shall have a responsible development and training officer who will oversee the training and skill enhancement activities.

The potential provider’s organization shall develop an annual plan for training and advanced training of personnel involved in ground operations, and ensure the implementation thereof.

All types of permits for ground personnel to perform ground operations shall be issued by an order of the head of the organization, provided they have documents confirming the successful completion of training and internship programs.

The ground personnel shall be issued with a document (certificate) for the right to conduct ground operations, which is mandatory for the performance of ground operations.

The potential provider shall maintain a record of ground personnel authorized to conduct ground operations.

The employees operating ground handling facilities or special machinery shall have a permit to operate such type of ground handling facilities or special machinery to be issued by an order of the head of the potential provider's entity following the relevant training. The permit for the right to operate a ground handling facility at the Almaty airfield shall be issued by the airport operator in accordance with the Airfield User Manual.

The employees operating the automobile transport or ground handling facilities shall undergo the mandatory medical control before the start and at the end of their work shift and have a supporting document during the work shift.

All personnel performing their official duties on ground handling shall be provided with the necessary personal protective equipment.

Requirements to the information and documentation support to the activities of a ground handling organization

The potential provider's organization shall have in place the information sharing system ensuring the sharing of information between management and the operating personnel that affects flight safety, aviation security and the quality of ground services.

In order to ensure the effective operation, the information sharing system shall cover the representatives of the airport operator, the airline and other ground handling organizations and the air navigation services organization.

The potential provider shall have a system in place to manage and control internal and external documentation, including data used directly during the ground operations.

The potential provider shall have at its disposal a set of documents, including:

- Laws of the Republic of Kazakhstan on civil aviation, as per paragraph 9 of this technical specification;

- Documents of a general and administrative nature, job and technological instructions (maps), service regulations;

- Ground Operations Manual, as required by the laws of the Republic of Kazakhstan and the IATA IGOM;

- Flight Safety Management Manual, as required by the ICAO (ICAO DOC 9859, ICAO DOC 10121);

- Aviation Security Program, as required by the current laws of the Republic of Kazakhstan;

- Documents containing the information on the daily scope of work, ensuring the safety and uninterrupted flights of the customer's aircrafts under agreements (contracts).

The Ground Operations Manual shall be developed in accordance with the requirements of the current legislation of the Republic of Kazakhstan, as well as the standards and recommended practices of the International Civil Aviation Organization (ICAO DOC 10121) and the aviation regulations of civil aviation international organizations (IATA IGOM), and shall be incorporated in their structure and content, subject to type of stated services:

- the organizational structure and management of the organization;

- procedure for personnel training and admission to independent work:

- technological processes for ground handling;

- technological schedules for ground handling of aircrafts by types and categories of flight handling;

- requirements to the operation, technical inspection and admission of ground equipment, special machinery and mechanization to work on the apron;

- ensuring the labor protection and safety;

- ensuring the fire safety;

- ensuring the flight safety;

- ensuring the aviation security;

- ensuring the quality of the provided ground handling services, including the regularity thereof.

Infrastructure Requirements

The potential provider shall have premises and facilities that meet the requirements for carrying out works in the stated area of activities, that ensure the protection against adverse weather conditions, and are suitable for arranging workplaces, accommodating personnel, ground handling facilities, tools, storage and amenity premises (own or leased).

Workplaces shall be equipped with necessary office appliances and computer hardware and meet the requirements to labor safety, fire safety and environmental protection.

The work environment shall be suitable for the works being done and allow the personnel to perform their work in an efficient manner.

The premises shall be adapted for the storage of ground equipment, tools and materials.

Communications (telephone communications, “GORE” warning system, radio communicationa), with the possibility of connection to the airport system.

Requirements to the special machinery and ground handling facilities

The organization of the potential provider shall have in place sufficient ground handling facilities, machinery, equipment, tools and materials, either own or leased, for the performance of works in the stated area of activities.

The potential provider shall provide a list of special machinery and ground handling facilities by year of manufacture, the number of moto/hours worked.

The potential provider shall provide a program of maintenance, repair of special machinery and ground handling facilities in accordance with the requirements of manufacturers and have in place the necessary technical base.

All special machinery and ground handling facilities shall comply with the requirements of the types of aircraft stated for servicing, the requirements of the current legislation of the Republic of Kazakhstan, as well as the safety standards set forth in IATA AHM 905A, and have a certificate of conformity.

Periods of preventive maintenance shall be determined by the operating instructions for the relevant special machinery or ground handling facilities. If no such period is defined, the preventive operations shall be carried out during seasonal preparations (SSP, AWP) and shall be documented.

The organization of the potential provider shall develop regulations for the maintenance of ground handling facilities based on certificates and operating instructions of the manufacturer of a particular type of equipment, the release on the road procedure for ground handling facilities (for operations at an aerodrome).

The potential provider shall have certified maintenance personnel or an agreement (with a certified organization) for the maintenance of special machinery and ground handling facilities.

The personnel engaging in the technical inspection and the on-road release of special machinery and ground handling facilities shall have the appropriate qualification of an engineer/mechanic.

Appendix 1
to the Technical Specification

Ground Administration and Control

Representation of an airline at the airport:

the service shall be documented and provided under the service provider’s GOM as per the IATA IGOM;

the availability of a specialist with at least 3-year experience as a representative of airlines, in the area of aircraft apron servicing or passenger servicing;

the availability of a vehicle equipped for movement on the airfield.

Baggage Handling

Baggage handling:

the service shall be documented and provided under the service provider’s GOM as per the IATA IGOM and the current legislation of the Republic of Kazakhstan;

employees engaged in baggage handling operations shall undergo practical and theoretical training on baggage completion, loading control, handing baggage of special categories;

baggage shall be distinguished by categories envisaged by the carrier:

priority baggage, the baggage that has priority for unloading and delivery to the place of release/sorting. Baggage with priority tags shall be delivered to the conveyor belt in the arrivals hall earlier than the other baggage;

checked baggage, the baggage that has priority for unloading and delivery to the place of release/sorting due to the short connecting time at the transfer point;

containers with live animals carried in the cabin or in the baggage section of an aircraft;

fragile baggage carried in the baggage section of an aircraft;

baggage delivered by a passenger himself/herself to the aircraft and received after landing at the place of destination (baby strollers, wheelchairs);

mishandled baggage;

the baggage handling shall be carried out in the baggage compartment of an airport using:

technical means for loading, unloading and transporting baggage and mail (baggage tractors, baggage carts and automatic conveyor belts with the baggage sorting system);

means of communications for sending electronic messages about baggage in the format used by airlines;

electronic reading devices that allow to scan a barcode and determine the belonging of baggage during loading and unloading;

an electronic system for informing passengers about the movement of their baggage in the terminal and about the expected time of releasing the first bag and the last bag on the belt (the display above the belt in the arrivals hall);

the potential provider shall have baggage carts of closed type or equipped with an awning.

The potential provider shall have:

the procedure for handling oversized baggage;

the procedure for releasing baggage to passengers on dispensable conveyor belts in the baggage compartments of the terminal;

the procedure for handling and storing unclaimed or mishandled transfer baggage;

the procedure for the release, registration, search and storage of unclaimed, forwarded, damaged, screened, non-documented, lost and exchanged baggage.

Apron Service

Meeting-release:

the service shall be documented and provided under the service provider's GOM as per the IATA IGOM;

English language skills at ICAO English Level lV or higher;

the personnel shall be equipped with an aircraft intercom, light-signal batons, mobile radio stations;

safety signal cones conforming to the IATA AHM in a quantity conforming to the IATA IGOM;

thrust pads conforming to the IATA AHM approved for use by the aircraft manufacturer in a quantity conforming to the IATA IGOM.

Transportation and loading/unloading of baggage, cargo/mail:

the service shall be documented and provided under the service provider’s GOM as per the IATA IGOM;

the service shall be provided in accordance with the Rules for Organizing Passenger Service at Airports of the Republic of Kazakhstan, approved by Order No. 189 of the Minister for Investments and Development of the Republic Kazakhstan dated 24.02.2015, the IATA DGR, the IATA Temperature Control Regulation, the IATA Perishable Cargo Regulation, the IATA Live Animal Regulation, the Cargo Handling Manual, the IATA Cargo Service Conference Resolution;

availability of apron mechanization facilities conforming to the types of aircrafts stated for servicing in a quantity that allows simultaneous unloading/loading of baggage and prompt delivery (belt loaders, container loaders, baggage carts, carts for containers and pallets, baggage tractors);

special vehicles and equipment shall be in technically sound condition.

Aircraft towing:

the service shall be documented and provided under the service provider’s GOM as per the IATA IGOM;

availability of means for towing an aircraft (towing tractor) conforming to the types of aircrafts stated for servicing, in a quantity sufficient for the stated scope of services;

towing carrier conforming to the types of aircrafts stated for servicing, for towing tractors;

a towing manager shall have at least 2-year experience in the aircraft apron maintenance and English language skills at ICAO Level IV;

a driver of the towing tractor shall have at least 2-year experience in driving a vehicle on the apron, and the appropriate training for the operation of the towing tractor;

special vehicles and equipment shall be in the technically sound condition;

it is necessary to have in place the approved procedure for emergency or unusual situations (rules for the technical operation and repair of aviation equipment).

Aircraft de/anti-icing treatment:

the service shall be documented and provided under the service provider's Aircraft Ground De-Icing Guidelines as per the current legislation of the Republic of Kazakhstan and SAE AS 6285, 6286, 6257, 6332 and ICAO DOC 9640;

employees engaged in the anti-icing treatment of aircrafts are required to undergo training in accordance with the requirements of the Rules as per SAE 6286 standards;

the service provider shall ensure the use of the de-icing liquid as per the requirements of the laws of the Republic of Kazakhstan and SAE 1424 and 1428, and included in the qualification lists of the FAA (Federal Aviation Administration) or Transport Canada;

the obligatory documents for de/anti-icing liquid (the “AIL”) include certificates of conformity of the liquid, opinions of the AMIL (Anti-Icing Materials lnternational Laboratory), included in the qualification lists of the FAA (Federal Aviation Administration) or Transport Canada;

availability of special machinery for anti-icing protection using anti-icing fluid (preferably the presence of an air blowing system) corresponding to the types of aircraft declared for maintenance in an amount that allows the aircraft to be treated by at least two special vehicles;

availability of special machinery for the heat treatment of aircrafts in case of snow and ice deposits, conforming to the types of aircrafts stated for servicing, in sufficient quantities;

special vehicles and equipment shall be in a technically sound condition;

an operator shall have the qualification and proven experience in anti-icing protection of aircrafts on the ground for at least one year and the English language skills at ICAO Level lV;

a driver of a special vehicle shall have at least 2-year experience in driving a vehicle on an apron, including at least one year of experience driving a special vehicle for anti-icing protection using AIL;

there shall be an approved procedure for emergency or unusual situations.

Aircraft Interior Cleaning

Aircraft interior cleaning:

the service shall be documented and provided under the service provider’s GOM and meet the requirements of aircraft manufacturers;

aircraft interior cleaning the shall be carried out at each landing of the aircraft and between flights during the long-term parking of the aircraft (subject to the flight schedule and the request for aircraft cleaning from the airline);

prior to proceeding with the provision of the services, the provider shall acquire the professional equipment (vacuum cleaners, etc.) for cleaning aircrafts with the ability to connect to the power supply (sockets) in an aircraft with voltage and frequency of 115V/400 Hz.

a number of employees engaged in the prompt cleaning shall correspond to a certain type of an aircraft and constitute at least:

up to 100 seats, 6 people;

up to 150 seats, 7 people;

up to 200 seats, 8 people;

up to 250 seats, 12 people;

over 250 seats, 15 people.

the availability of premises (own or leased) on the territory of the airport for the operating personnel;

availability of vehicles (own or leased) that meet the requirements for equipping vehicles (marking, flashing beacon, means of communications, sufficient space for transportation of personnel and aircraft cleaning tools, and other necessary requirements as per the Kazakhstan laws and the internal documents of the provider);

availability of a trailer cart (open type) for transportation of garbage packed mandatorily in plastic bags;

production personnel shall be provided with clothes of uniform design with distinctive signs;

cleaning equipment should be marked depending on the premises to be cleaned (buffet-kitchen, salon, toilet);

special vehicles and equipment shall be in a technically sound condition;

availability of detergents and disinfectants and equipment approved by the aircraft manufacturer;

availability of premises for the storage of detergents and disinfectants, consumables, airline service materials, storage of inventory, preparation of washing / disinfecting solutions;

technical rooms shall have sewerage for draining the used solution;

a driver of the vehicle shall have at least one year of experience in driving a vehicle on an apron.

In-Flight Catering Supply

GH organizations that supply in-flight meals shall meet the following requirements:

the service shall be documented and provided under the service provider’s GOM compliant with the IATA IGOM;

a forwarder of the special catering vehicle shall have the appropriate qualifications and work experience as a forwarder for at least one year:

a driver of the special in-flight catering vehicle shall have experience in driving a vehicle on the apron for at least one year, have appropriate training for the operation of a special in-flight catering vehicle;

availability of special on-board catering vehicles conforming to the types of aircrafts stated for servicing in a quantity sufficient for the stated scope of services;

the special on-board catering vehicle shall be in a technically sound condition.

the supplier shall ensure that in-flight meals and in-flight supplies pass through security controls;

all vehicles with in-flight catering with on-board supplies shall be sealed when leaving the in-flight catering workshop;

approach/departure to/from an he aircraft shall be made with the permission of a person responsible for ground operations and under the guidance of a person responsible for approach/departure;

a procedure for loading the on-board meal on board of an aircraft shall be developed.

the special machinery shall be delivered to an aircraft in accordance with the requirements of the carrier, within specified periods of time.